EEE 591

LAB #04

- Full adder schematic and sizing

- Nmos:300nm

- Pmos:457nm

  - Explain the sizing criteria used

The minimum sized NMOS transistor you can use has WN = 300nm;I choose the mirror adder because it is easier to size the pmos transistor;all the pull-up network and pull-down network are symmetric;so I can sweep the pmos transistors sizes and get the suitable sizes to let Wp to match the LH and HL delay.

  - Provide the sizes (W/L) of all your transistors

- All the nmos transistors:300nm;      All the pmos transistors:457nm

  - Provide the schematic of your 4-bit design

  - Provide any scaling you choose to do for the 4-bit design and justification for the choice

- I put the inverter outside the 1-bit adder design,and I just add the inverter to the Cout;I only have 5 inverters in my design.And it is easy to design and layout for the mirror_adder.

- ·Simplicity: The ripple-carry adder is easy to design and implement. It simply consists of a series of full adders (FA), each responsible for adding a bit from each operand along with a carry bit from the previous stage.

- ·Low Hardware Cost: Due to its simple design, the ripple-carry adder requires fewer components compared to other types of adders like carry-lookahead adders. It is cost-effective in terms of area and power consumption.

- Simulation results

  - Include the worst case delay plots

- Worst case delay = 231ps

  - Include the power consumption analysis results

- Power=21.784uw

- Include confirmation of

  - Layout & DRC Pass

  - Layout & LVS Pass

- Report the total area of your layout

- Width = 9.624um

- Height =1.7um

- Area = width * height = 16.3608um^2

- Report your overall score as outlined in the lab document

| - Delay | - 231ps |
| --- | --- |
| - Power | - 21.784uw |
| - Area | - 16.3608um^2 |
| - Score | - 15.5 |

1-bit adder layout

4-bit adder layout